# Rice Yield Modelling using Vegetation Indices of Landsat-8 and Radarsat-2 Satellite Imagery

Siti Aishah Mohammad Rasit (1, 2), Abdul Rashid Mohammed Shariff (1, 2), Aimrun Wayayok (2), Ahmad Fikri Abdullah (2)

1 Laboratory of Plantation System Technology and Mechanization, Institution of Plantation Studies, Universiti Putra Malaysia, 43400 Serdang, Selangor.

2 Department of Biological and Agriculural Engineering, Faculty of Engineering, Universiti Putra Malaysia, 43400 Serdang, Selangor.

Email: aisyahrasyid1207@gmail.com

KEY WORDS: Rice yield modelling, vegetation indices (VI's), backscatter coefficients (σ°), Landsat-8 OLI & RADARSAT-2.

ABSTRACT: Rice is the most important food crop which is the source of fiber and carbohydrate. In Malaysia rice productivity is steadily increasing and Malaysia’s self-sufficiency level of rice is at 72%. Quantification of rice parameters, such as Vegetation Indices (VIs) and Backscatter Coefficients (σ°) is important in rice yield productivity modelling. Yield modelling is the best tool for rice crop and productivity management. Various VIs and backscatter coefficients are being used to monitor crop growth and to predict and estimate yield before harvesting date. Previous studies shows most analysis use satellite images close to harvesting date of rice crop. The aims of this study is to develop rice yield modelling through on VIs i.e. Normalize Difference Vegetation Index (NDVI), Soil Adjusted Vegetation Index (SAVI), and Leaf Area Index (LAI) and backscatter coefficients (σ°) for 30 plots of rice fields in Sawah Sempadan, Selangor, Malaysia rice granary area. Other purposes of this study are to examine the correlation of the VIs and σ° with the reported yield and study the effectiveness of using Optical and SAR data for rice yield modelling. The first method of this modelling involves data processing of Landsat 8 OLI and Radarsat2 imageries to obtain the VIs of rice crop and the second part present regression analysis of VIs with the reported yield of rice crop. The yield prediction equation gives good correlation (R2) result for LAI which is R2=0.81, compared to NDVI (R2=0.61) and SAVI (R2=0.71) for Landsat 8 dataset. While the analysis of Radarsat-2 gave very low correlation (R2=0.10). In this limited study, it was observed that SAVI gives a better result for modelling yield at the time of the productive stage.

# INTRODUCTION

## 1.1 Research Overview

Rice is known as Oryza Sativa L (Inoue et. al, 2014) and it is the most important food crop which is the source of fiber and carbohydrate. In Malaysia, rice productivity is steadily increasing and Malaysia’s self-sufficiency level of rice is at 72%. Rice provides main economic profit for Malaysia's domestic market after pineapple, bananas, and many more. Asian countries responsible for around 90% of the world rice productivity and consumptions, especially in China, India, and Indonesia. According to Radin Firdaus et al., (2012), in Malaysia, rice is mostly planted in Peninsular Malaysia and contributes approximately 85.5% of Malaysia’s total rice productivity. In 2014, Malaysia rice production is about 2.65 million tones with total area of rice cultivation of 689, 732 hectare rice field (MOA, 2014). Because of the limited land, Malaysia does not produce maximum potential of high rice yield (MARDI, 2014), but Malaysian rice varieties contributes higher yields by a lesser amount of compost and fertilizer.

In order to farm sustainable while still maximizing land productivity and managing input costs of farmers it will need more information on the crops. Toward increasing the rice production in terms of food security in times of climate change and rapid development is a big challenge. Rice’s productivity in the future is unsure as climate change affects water resources and temperature (Tiara et al 2015). Due to climate change, weather conditions and temperatures have slightly affected the presence of various diseases and pest infestation of paddy rice. Rice pests and diseases are negatively affected the rice production in Malaysia.

The proficiency in satellite-based rice yields forecasting before harvesting remains a significant factor in several aspect for decision making in agricultural as it enables the management to change the farming practices during the next growing season in order to exploit the yield productivity and increase the profit while reducing the cost. The reflectance band from rice canopy and vegetation indices (VIs) derive from optical data along with radar backscatter from SAR data were used to produce the rice yield forecasting models.

## 1.2 Research Problem

Rice’s growths monitoring, mapping and yield estimation approaches based on traditional method remains used in data gathering, and rice crop information because the data accurateness is uncertainty and due to limitation in local scale, especially regarding to be planted and harvested area. In this paper, we measured the yields across the temporal and spatial scales. The advantage of earth observation by satellites can collect such information of over a broad area, even if the area is difficult to access, periodically, with high consistency, in near real‐time, and cost‐effective, (Shin-ichi, et al., 2014).

In rice modeling, there is a challenge in estimating rice cultivation area and yield using EO data is to obtain well-timed satellite data, which enables to interpreted rice plant with other crops, and land uses figures. Monitoring rice cropping need multi-temporal data set because the complex rice environment had at least three stages of growing phase in a season. Meaning that, it needed enough images during the rice-growing season. Refer to Shin-ichi (2014), multi temporal of satellite observation, together with international constellation is essential because agriculture needs multiple sources of data integration. With only one sensor or satellite, it does not meet the analysis and application requirement. A precise crop yield forecasting model would be able to support managers or farmers to make the best decisions in farming practices.

## 1.3 Objective

The aim of this study is to develop a prediction model of rice yield using three different vegetation indices; NDVI, LAI, and SAVI from Landsat 8 sensor and radar backscatter coefficient from Radarat-2 at the stage of productive phase. We also examine the correlation of vegetation indices with reported yield to study the effectiveness of using SAR data for rice yield forecasting model.

# Materials and Methodologies

## Study Area

The study area was in Sawah Sempadan C2 Block of Tanjong Karang (Figure 1 (a) and (b)) rice granary area under monitoring of Integrated Agricultural Development Area (IADA), Barat Laut Kuala Selangor. IADA Barat Laut Selangor was under Ministry of Agriculture and Agro-Based Industry Malaysia, and it is the third major rice production in Malaysia after MADA in Kedah and KADA in Kelantan. Rice’s cultivation one of the significant sectors of Malaysia's economic source after oil palm and rubber that contribute about 12% to the domestic GDP. According to Che Omar et al, (2019), rice harvested area remained relatively stagnant compare to oil palm harvested area that increased over the years. This sector offered employment opportunities to the public.

In 2016, IADA Barat Laut Selangor generally produced 222,033 MT with 5.8 MT/Ha (hectare), which is the rice production is above national average of 4.0 MT/Ha (Che Omar et al., 2019). The area of Sawah Sempadan covering 2303km2 with the population density of 1202 people. Rice’s cultivation in the region is depended on the irrigation system which consists of two main rice cropping seasons as Sawah Sempadan started in July to November for the first season and Jannuary to May for the second season. The rice planting will follow the irrigation schedule to make sure all the rice planting area in Tanjong Karang region will get sufficient water supply for paddy growth.

- (b)

Figure 1: Location of the study site at different scene of (a) Landsat 8 OLI imagery and (b) RADARSAT-2 imagery.

## 2.2 Satellite Data Description

Table 1: Data specification and description of Landsat 8 OLI and RADARSAT-2

| Specification | Description |
| --- | --- |
| Operating Sensor | Operational Land Imager (OLI) |
| Wavelengths (Multispectral) | 0.43-1.38 |
| Number of Bands | 9 |
| Resolutions | 30m (Band 1-7) and 15m (Band 9) |
| Datum | WGS 84 |
| Map Projections | UTM Zone 47 N |
| WRS-2 Path /Row | 127/58 |
| Corner LL Lat/Long Product | 1.83972/100.32521 |
| Corner LR Lat/Long Product | 1.83700/102.37877 |
| Corner UL Lat/Long Product | 3.94259/100.33575 |
| Corner UR Lat/Long Product | 3.93676/100.33330 |
| Actual Size of Scene | 170km x 183km |
| Date Acquired | 2015-09-19 |
| Pan-sharpening Used | Gram-Schmid |
| Operating Sensor | RADARSAT-2 |  |
| Beam Mode | Wide Fine (W0F1) |  |
| Product | SGX, SGF & SLC |  |
| Nominal Pixel Spacing (m) | 3.13 x 3.13 |  |
| Spatial Resolution (m) | 12.5 |  |
| Nominal Scene Size (km) | 150 x 150 |  |
| Nominal Incidence Angle Range (0) | 200 to 450 |  |
| No. Looks | 1 x 1 |  |
| Band | C-band |  |
| Mode | Ascending |  |
| Polarization | VV |  |
| Date Acquired | 2015-05-09, 2015-06-02, and 2015-08-13 |  |

### 2.3 Data Processing

Landsat 8 OLI data processing was performed using ENVI and ArcGIS software. While, RADARSAT-2 data processing was performed using open-source software, Sentinel Application Platform (SNAP) that was developed by European Space Agency (ESA) for earth observation. The flow of data processing is shown as in Figure 2 below;

Figure 2: Landsat-8 and RADARSAT-2 data processing outline.

## 2.3.1 Radar Backscatter Coefficient

Backscatter coefficient (𝜎°) was derived from RADARSAT-2 C-band, vertical transmit and vertical receive (VV) polarization using equations below;

σ°=10*logDNjk2+ A°Aj+10*logsinIji                                                                                                   (1)

Where; σ0 is the sigma naught or backscattering coefficient (dB). DN is the digital number (or raw pixel value) of RADARSAT-2 image, Ao and Aj are the automatic gain control factor, and Ij is the local incidence angle of each pixel across the range direction.

## 2.3.2 Vegetation Indices

Three VIs were intended from different forms arithmetical ratios concerning red (r) and infrared bands (ir); Normalized Difference Vegetation Index (NDVI), Soil Adjusted Vegetation Index (SAVI), and Leaf Area Index (LAI). All the VIs were calculated using equations as follows:

NDVI = ρir-ρrρir +ρr                                                                                                                                          (2)

NDVI was introduce by Rouse et al. in 1973. Where ρir the reflectance of infrared band (band 4 and band 5) and ρr is the red band reflectance (band 3 and band 4).

SAVI= (ρir - ρr )ρir + ρr + L x 1+L                                                                                                   (3)

SAVI was introduce by Huete (1998) to minimize soil brightness and it was set to 0.5 for OLI.  Where L is a constant that is a surrogate for LAI as the optimal adjustment factor are;

L = 1 for the low vegetation density (low dense such crop vegetation; rice)

LAI= [Leaf area (m2)/Ground cover (m2)] 							         (4)

LAI is a measurement of canopy growth at a specified time. Referring to Ahmad, et al. (2015), direct measurement of LAI involved the destructive and non-destructive methods. Destructive method requires leaf sample collection for further laboratories’ analysis, and this method would give high accuracy. Non-destructive method requires field measurement of leave (width and length). According to Bange et al. (2000) field linear dimension of leave is a well-established method and had been widely used to measure LAI non-destructively. Chanda and Singh (2002), use equation a=b*Max length*width for linear measurement of leave where b is a coefficient. Although, Quarrie and Jones use equation, LA= Length*Breadth*0.75 for wheat crop (Aldesuquy et al., 2014).

LAI also can be derived from satellite imagery either optical or SAR sensor as indirect measurement. According to Cohen et al. also supported in using VIs from satellite imagery through regression analyses LAI estimation. Meyer et al. (2019), use Sentinel-2 MSI and Landsat-8 OLI in estimating LAI. According to Blinn et al.  (2019), ground LAI was estimated based on minimum and maximum LAI and modeled with Landsat-derived VIs from surface reflectance, simple ratio (SR) and Normalized Difference Moisture Index (NDMI).

## 2.4 Yield Prediction Algorithm

ML is a significant algorithm for crop yield prediction to help managers and farmers for decision support in farming practices of the growing season for the rice crops (Thomas et al, 2020). Numerous ML algorithms have been used in modeling crop yield. Based on the previous and current work, the Neural Network and Regression algorithm were employed mostly to predict the grain yield (Aditya Shastry et al. 2017; Thomas et al, 2020). This study will apply Single Linear Regression (SLR) and Multi Linear Regression (MLR) model to generate yield modeling for rice crop based on VIs from optical data and σ° from SAR data.

# RESULT AND ANALYSIS

## 3.1 Single Linear Regression (SLR)

Generated SLR formula and correlation coefficient of rice yield, as a dependent variable, between VIs (NDVI, SAVI and LAI) from Landsat-8 OLI as independent variables, it is shown that the appropriate mathematical linear were uniformed as shown in Figure 3: (a), (b), and (c). According to regression equations (Table 2), there is a typical relationship between rice yield either with NDVI, SAVI and LAI with the R2= 0.61; 0.71; and 0.81 except with the case of 𝜎° model from single RADARSAT-2 data shows the lowest determination coefficient or the R2 values which is 0.31 and it consider as very weak as shown in Figure 3(d). LAI model approximately R2 = 0.81 representing high correlation with the rice yield which means the value considered strong effect size because the R2 value is above 0.7. This result indicates LAI and rice yield have good correlation, and as a result the retrieval equations performed well as shown in Figure 3(c). According to Blinn, et al. 2019, Landsat OLI generates maximum accuracy LAI model for a specified time period with the simple ratio (SR) vegetation index.

## 3.2 Multi Linear Regression (MLR)

Multi Linear Regression (MLR) model and determination of coefficient were executed based on the combination of yield and radar backscatter coefficients values (𝜎°) that was obtained from RADARSAT-2 of different date which is the rice was in late maturity and ripening stages. Prediction accuracy of regarding MLR formula and the R2 for 𝜎° (Table 2), was enhanced to compare to SLR formula which R2 is 0.62 and 0.13 respectively. The efficiency increase when adding the more independent variables to the prediction model. The coefficients of 𝜎° model strangely have the low amount of explained variation. This could be due to the quality of the data, the selection of ML or the parameter input. SAR data was analysed by considering two main aspects, which is noise equivalent sigma zero (NESZ) and the radiometric resolution (Fiuezal, et al. 2017).

The signal noise ratio (SNR) determine the quality of SAR data. Noise is in fact error in the rendering of the imaged scene, and is due to a variety of phenomena from a variety of sources (Armin, 2006). The range of 𝜎°VV C-band is from -10dB to -6dB after processing and extracting. This is quite low compare to another polarization. Agreeing to Brisco e al. (2008), the VV polarisation comparatively not good as HH, due to low 𝜎°, thus decreasing the sensitivity to soil moisture and this enlarged the sensitivity to microscale surface roughness due to low SNR. The lower value of 𝜎°, the lower SNR. According to Johansson et al. (2018) variations in thickness and surface roughness strongly affect the backscatter signature. Consequently, the 𝜎° of rice area should be higher due to the variations in thickness and surface roughness of the rice canopy surface. Single data of co-polarization (VV) is unsuitable for vegetation application but perfect for ocean applications. This theory was verified by Toure´ et al. (1994) in the sensitivity analysis using theoretical surface scattering models.

Table 2: Regression models for rice yield prediction using single optical and multi-temporal SAR

| Algorithm/Forecaster |  | Model | R2 |
| --- | --- | --- | --- |
| Single Linear Regression |  |  |  |
| NDVI | Y=69934*NDVI-54177 | 0.61 |
| SAVI | Y=53340*SAVI-62983 | 0.71 |
| LAI | Y=5427.4*LAI-37532 | 0.81 |
| 𝜎°vv | Y=-500.67*𝜎°Re+1905.1 | 0.13 |
| Multi Linear Regression |  |  |  |
| 𝜎°vv |  | Y= 1013.4025 *𝜎°LM + (-321.735)*𝜎°Ri +11786.9378 | 0.62 |

𝜎°: Backscatter Coefficients

R2: Determination Coefficients

Re: Reproductive

LM: Late Maturity

Ri: Ripening

- (b)

(c)                                                                          (d)

Figure 3(a-d): Relationship between VIs and Backscatter Coefficients (𝜎°) with reported yield.

# CONCLUSION

Through the first analysis, it demonstrates that the yield modeling from Landsat-derived VIs (LAI) shows the best performance with 0.81 determination coefficient using SLR model. This method offers the advantage of using open source of satellite image from Landsat imager, while presenting good results compare with other complex techniques. In contrast, the radar backscatter produce very weak results with single data but the accuracy improve (R2=0.62) when combining two variables (𝜎°VV C-band from multi dates).

For satellite SAR approach, it is needs all the imageries acquired all over the entire rice crop growth to enhance the accuracy of the prediction model. However, due to weakness of VV polarization, it is recommended to use HH polarization or HV to reduce the effect of SNR. By combining the optical and SAR data together with Vis or other variables (crop, soil, weather and nutrient parameter), it potentially can produce very high accuracy of rice yield modeling. This finding demonstrates the advantage of using satellite data in modeling the rice yield with simple method and the poor interest of data acquisition as the independent variables for model development. Upcoming work will be expending more high-resolution satellite data and accumulate more variables to gain the accuracy assessment.

# AKNOWLEDGEMENT

This study was funded by Geran Putra Siswazah UPM (GP-IPS) under research project, Rice Monitoring, Estimation and Damage Assessment in Kuala Selangor using Geospatial Technologies and ScFund Grant from Ministry of Science and Technology (MOSTI) under research project, Synthetic Aperture Radar (SAR)-based Analysis of Rice Pest Infestation and Plant Stress Identification. The RADARSAT-2 data was delivered by Canadian Space Agency under the Asia-RICE Project (SOAR-JECAM Program) while the Landsat-8 OLI image was acquired from European Space Agency (ESA).

# REFERENCES

Aditya Shastry, H.A. Sanjay and E. Bhanusree. 2017. Prediction of Crop Yield Using Regression Techniques. International Journal of Software Computing, Vol 12, 2, 96-102. http://dx.doi.org/10.36478/ijscomp.2017.96.102

Ahmad, S., Ali, H., Rehman, A.U., Zeb   Khan, R.J., Ahmad, W., Fatima, Z, Abbas, G., Irfan, M., Ali, H., Azam,  M.K., and Hasanuzzaman, M.  2002. Measuring Leaf Area of Winter Cereals by Different Techniques: A Comparison. Pakistan Journal of Life and Social Sciences. 13(2): 117‐125

Aldesuquy H, Z Baka and B Mickky, 2014. Kinetin and spermine  mediated  induction  of  salt  tolerance  in wheat plants: Leaf area, photosynthesis and chloroplast  ultrastructure  of  flag  leaf  at  ear  emergence.   Egyptian   Journal   of   Basic   and   Applied Sciences, 1: 77-87

Armin. W. D. 2006. Performance Limits for Synthetic Aperture Radar. Sandia Report. Sandia National Laboratories Albuquerque, New Mexico and Livermore, California.

Bange  MP,  GL  Hammer,  SP  Milroy  and  KG  Rickert,  2000.  Improving estimates of individual leaf area of sunflower.  Agronomy Journal, 92:  761-765.

Blinn, C.E., House, M. N., Wynne, R. H., 1, Thomas, V. A., Fox, T.R., and Sumnall, M. 2019. Landsat 8 Based Leaf Area Index Estimation in Loblolly Pine Plantations. Journal of MDPI, Remote Sensing. 10(3), 222. https://doi.org/10.3390/f10030222

Brisco, B., Touzi. R.., Van der Sanden, J.J., Charbonneau, F., Pultz, T.J. and D'Iorio, M. 2008. Water resource applications with RADARSAT-2 – a preview, International Journal of Digital Earth, 1:1, 130-147. https://doi.org/10.1080/17538940701782577

Centre d’Études de la BIOsphère (CESBIO), Université de Toulouse, CNES/CNRS/IRD/UPS, Toulouse, France.

Chanda, S.V. and Singh, Y.D. 2002.  Estimation of leaf area in wheat using linear measurements. Plant Breeding and Seed Science, 46: 75-79.

Che Omar, S., Shaharudin, A., and Tumin, S. A. 2019. The Status of the Paddy and Rice Industry in Malaysia. Kazanah Research Institut. Kuala Lumpur, Malaysia.

Cohen, W.B.; Maiersperger, T.K.; Gower, S.T.; Turner, D.P. An improved strategy for regression of biophysical variables and Landsat ETM+ data. Remote Sens. Environ. 2003, 84, 561–571.

Fatimah, Mohd Arsyad., Mohd Fauzi, Jani. & Mohd Khanif, Yusop. 2010. Agenda Polisi Sekuriti Makanan Malaysia. Bengkel Mengarus perdana Pertanian dalam Model Baru Ekonomi Malaysia, Majlis Professor Negara, Bangi, 9-10 November 2010.

Fieuzal. R., Marais Sicre, C., and Baup, F. 2017. Estimation of corn yield using multi-temporal optical and radar satellite data and artificial neural networks. International Journal of Applied Earth Observation and Geoinformation Volume 57, Pages 14-23. https://doi.org/10.1016/j.jag.2016.12.011

Inoue, Y., Sakaiya, E., and Wang, C. 2014. Potential of X-Band Images from High-Resolution Satellite SAR Sensors to Assess Growth and Yield in Paddy Rice. Journal of Remote Sensing, 6 (7): 5995-6019.

Huete, A.R. A soil-adjusted vegetation index (SAVI). Remote Sens Environ. 1988, 25, 295–309

Johansson A. M., Brekke, C., Spreen, G. and Ki, J. A. 2018. X-, C-, and L-band SAR signatures of newly formed sea ice in Arctic leads during winter and spring. Remote Sensing of Environment Volume 204, Pages 162-180. https://doi.org/10.1016/j.rse.2017.10.032

Meyer L. H., Heurich, M., Beudert, B., Premier, J., and Pflugmacher, D. 2019. Comparison of Landsat-8 and Sentinel-2 Data for Estimation of Leaf Area Index in Temperate Forests. Journal of MDPI. Remote Sens. 2019, 11(10), 1160. https://doi.org/10.3390/rs11101160

MOA. 2014. Agrofood Statistics. Ministry of Agriculture & Agro-Based Industry Malaysia.

Rouse, J.W., Jr.; Haas, R.H.; Deering, D.W.; Schell, J.A.; Harlan, J.C. Monitoring the Vernal Advancement and Retrogradation (Green Wave Effect) of Natural Vegetation, NASA/GSFC Type III Final Report; Texas A&M University: College Station, TX, USA, 1974; 371p

Van Klompenburg, T., Kassahun, A. and Catal, C. 2020. Crop yield prediction using machine learning: A systematic literature review. Computers and Electronics in Agriculture. Volume 177, 105709. https://doi.org/10.1016/j.compag.2020.105709