SPECTRAL SIGNATURES IN LANDSAT 8 OLI IMAGE AND THEIR INTERPRETATION FOR LAND COVER STUDY

DUONG Nguyen Dinh, ANH Le Van, THU Ho Le,  ANH Nguyen Kim

Institute of Geography, Vietnam Academy of Science and Technology,

18 Hoang Quoc Viet Rd., Cau Giay, Hanoi, Vietnam

Email: duong.nguyen2007@gmail.com

KEY WORDS: Spectral signature, spectral reflectance curve, Landsat 8 OLI, modulation pattern.

ABSTRACT: Spectral signature, in general, can be defined as characteristics of surface objects in transmission, absorption and reflection of electromagnetic radiation. Spectral signature is expected stable and unique for given surface material. Spectral signature has been used long time for object detection and classification. The Landsat 8 OLI image data with six reflective bands and 30 m spatial resolution is excellent data source for land cover mapping in both local and global scale. Due to very huge data volume, automated analysis is crucial if land cover map is to be achieved in short time. In this paper, the authors introduce a concept to use modulation pattern of spectral signatures for understanding land cover. The first step in this concept is to understand correctly meaning of each modulation pattern of spectral signature and its relation to land cover class. Three Landsat 8 OLI images over Vietnam in 2013 and 2014 have been used in this research. Ground GPS field photos were collected to support land cover interpretation.

1. INTRODUCTION

Spectral signature is one of basic concepts of spectral remote sensing from the beginning. Spectral signature defines unique correspondence between a material and its reflectance (Shaw et al., 2003). Two-dimension plot of spectral signature is spectral reflectance curves (SRC), its shape is formed by varying reflectance amplitude. It is hypothesized that spectral reflectance curve of similar features have similar shapes. Therefore, shape of spectral reflectance curve has been exploited in detection algorithm where more weight is given to the shape than to the amplitude (Shaw et al., 2003).

In general, to detect a ground object, the SRC is to be compared with the one in spectral library. The comparison can be done by quantitative or qualitative spectral matching techniques (Thenkabail et al., 2007; Ruby et al., 2002). Obviously, without reference spectra, no matching technique can be executed. Matching is pixel-based algorithm with complicated computation of various variables like spectral correlation similarity, Euclidian distance similarity, spectral similarity value or spectral angle similarity etc. This computation is time consuming and have to repeatedly carried out for each pixel with all reference spectra in library. Evidently, if the matching is implemented with few reference spectra only, the analysis process could be much accelerated.

Because the shape of SRC is stable to land cover type so once the relation is established it can be used for ever. Shape of SRC is usually expressed in graphical form (curve) and it is really hard to express analytically relation between SRC and land cover type. There have been few reports about effort to encode binary the shape of SRC and its application in land cover mapping. The central thesis of this paper focuses to binary encoding of SRC shape and their interpretation.

In this paper, the authors develop a method to group spectral signature into patterns of SRC modulation, which can be used for explanation of land cover type associated with those patterns. The study pointed out that meaning of SRC modulation patterns is stable and they can be used for quick interpretation of land cover. Research has been implemented with six reflective bands of Landsat 8 OLI image data for three regions of Vietnam.

2. METHOD

2.1 Study Area

To illustrate method we developed here, we use three Landsat 8 OLI images as given in Table 1. The selected three OLI scenes represent typical geographic regions of Vietnam (Figure 3). The northern area – scene number LC81270452013336LGN00 is dominated by evergreen vegetation cover, paddy field and developed land (Hanoi City). The other two scenes in the central part of Vietnam cover high land region, which features both evergreen and deciduous forest, industrial tree plantation (coffee, rubber etc.). The scene LC81240522014046LGN00 includes also coastal zone with sandy terrain and semi-arid landscape cover. All the three scenes were acquired during dry season. Cloud coverage was in its low level so most of the area is under clear sky and ground objects were very well interpreted.

Table 1. Used Landsat 8 OLI scenes

| No | Scene ID | Path/row | Date of observation |
| --- | --- | --- | --- |
| 1 | LC81270452013336LGN00 | 127/045 | 2/12/2013 |
| 2 | LC81240512014030LGN00 | 124/051 | 30/01/2014 |
| 3 | LC81240522014046LGN00 | 124/052 | 15/02/2014 |

Figure 1. Location of the Landsat 8 OLI scenes over Vietnam

2.2 SRC Modulation Pattern Concept

In this research, we use six reflective bands of the OLI sensor and they are blue, green, red, near infrared, short wave infrared SWIR 1 and short wave infrared SWIR 2. These bands are named respectively as band 1, 2, 3, 4, 5 and 6 with abbreviations b1, b2, b3, b4, b5 and b6. Values of these variables are counted in reflectance by conversion of digital numbers DN to reflectance using gain and offset given in the metadata file of each Landsat OLI scene.  Each pixel vector has six values and can be written as p = b1, b2,b3,b4,b5,b6 where bi are reflectance value in band i. Spectral remote sensing expects that different surface materials are featured by different pixel vectors and objects of similar surface material are represented by similar pixel vectors. Two-dimensional plot of pixel vector where one axis shows reflectance value and the other displays spectral band number defines so called spectral reflectance curve SRC. Figure 2 shows example of spectral reflectance curves of water, vegetation, bare land and cloud. SRC are plotted by short wave infrared color composite RGB: 5, 4, 3.

Figure 2. Spectral reflectance curve of cloud (white), barren land (pink), vegetation (green) and water (dark blue)

As seen on Figure 2, different surface objects have different SRCs. These curves differ in different values of reflectance in each bands and result curves with different shapes. It is quite challenging to formulate shape of each reflectance curve in binary form. The authors develop a system for encoding the spectral reflectance curves. Duong (1997) has reported similar effort when conducted research with ADEOS-II GLI data. The encoding scheme is pairwise and relies on comparison of relative positions of two vertices. When two vertices have the same position (equal reflectance value), the score is 1. If position of one vertex is higher than position of the other vertex, the score is 2 and if position of one vertex is lower than the other, the score is 0. By this way, each pixel vector can be encoded by a series of digits 0, 1, 2 depending on relative position among vertices. To encode modulation of spectral reflectance curve, which has n vertices, we need nn-12 digits. In case of Landsat 8 OLI with 6 reflective bands we need 15 digits in total. Let us use mi,i+1 as position value between vertex i and i+1 then for Landsat 8 OLI pixel vector we have 15 digits for encoding of SRC modulation.

m1,2m1,3m1,4m1,5m1,6m2,3m2,4m2,5m2,6m3,4m3,5m3,6m4,5m4,6m5,6

Using this formula to encode SRC in Figure 2 we can obtain the following results:

Water pixel vector (9.2, 6.8, 4.8, 3.0, 0.8, 0.4), SRC code: 000000000000000

Vegetation pixel vector (8.6, 7.6, 5.4, 28.0, 15.4, 7.7), SRC code: 002200222222000

Barren land pixel vector (11.4, 12.8, 16.6, 22.0, 30.8, 22.8), SRC code: 222222222222220

Cloud pixel vector (48.8, 50.6, 54.6, 65.6, 55.4, 44.6), SRC code: 222202220220000

Algorithm for SRC modulation encoding is given bellow:

Declare integer*1 array modulation with 15 elements

Set array modulation to 1

While not the last pixel vector of the image

Set variable k=0

{

for (i=0;i<5;i++)

{

for (j=i+1;j<6;j++)

{

if(pixel vector [j]<pixel vector[i]) set array modulation[k] to 0

if(pixel vector [j]>pixel vector[i]) set array modulation[k] to 2

}

}

k++

}

Because the modulation pattern of SRC is composed of 15 digits, which exceed any integer value range in 32-bit programming environment so it is recommended to use either string variable or integral array to keep the patterns or switch to 64-bit programming environment. Theoretically, there are 3 power 15 or 14348907 possible modulation patterns for dataset with 6 bands. However, in practice the number of modulation patterns is fewer and depends on local land cover condition.

According to spectral reflectance characteristics of land cover, we can expect that modulation pattern 000000000000000 represents clear water because reflectance of water reduces with increasing wavelength. Modulation pattern 222222222222222 corresponds to dry bare land due to spectral reflectivity growth as wavelength increases. Modulation pattern 002200222222000 is typical for green vegetation (forest) because of slight high reflectance in green, low reflectance in red band and peak in near infrared band. This characteristic can be recognized when modulation pattern is filled by 2 in middle part of the pattern. After conversion of pixel vectors to modulations patterns, we can construct histogram to show their frequency. The modulation patterns are sorted from code 000000000000000 to 222222222222222 or by other words from water to bare land region. Shape of SRC modulation pattern histogram varies from area to area depending on land cover conditions.

LC81270452013336LGN00

(b)

(a)

LC81240522014046LGN00N00

LC81240512014030LGN00

(d)

(c)

Figure 3. Layout of SRC modulation pattern histogram (a), SRC modulation histograms of Landsat 8 OLI image:  LC81270452013336LGN00 (b), LC81240512014030LGN00(c), LC81240522014046LGN00 (d)

Figure 2 shows histogram template and histograms of modulation patterns for three Landsat OLI image used in this study. By sorting the SRC modulation patterns from water to barren land region, left site of the histogram is water; near center part is devoted to vegetation and right part to barren land. Histograms of major SRC modulation patterns of the three Landsat OLI images provide information not only about frequency of modulation patterns in each image but also about general land cover situation in each image. By studying the histogram, we can estimate percentage of water, vegetation and barren land in each image.

3. RESULTS

To understand relation between SRC modulation pattern and land cover types we decompose the OLI image data to component image with pixel vector of similar spectral signature or with the same SRC modulation pattern.

There are 559 SRC modulation patterns in the scene LC81270452013336LGN00. Pattern 002200220222000 has highest frequency with value of 13,570,460, which is equivalent to 32.69 % of pixels in the scene. There are many SRC modulation patterns with very small frequency. Table 2 show distribution of SRC modulation patterns for all three images. Only SRC modulation patterns, which have high enough frequency values, will be taken into account. For the scene LC81240512014030LGN00, there are 725 SRC modulation patterns in total. However, only 26 SRC modulation patterns dominate with 40,976,765 pixels corresponding to 98.57 % pixels in the image. Similar result can be achieved with the scene LC81240522014046LGN00. There are 674 SRC modulation patterns in total in this image, among them 25 SRC modulation patterns occupy 98.77% of total pixels equivalent to 41074629 pixels.

Table 2. Dominating SRC modulation patterns in the three Landsat 8 OLI images

| Landsat 8 OLI scene | SRC modulation patterns | Number of pixels | Percentage | Land cover type |
| --- | --- | --- | --- | --- |
| LC81270452013336LGN00 | 000000000000000 | 847910 | 2.04 | Water, wetlands |
|  | 000000000200000 | 267380 | 0.64 | Water, wetlands |
|  | 000000200200000 | 680690 | 1.64 | Cloud shadow, shadow, thin cloud, wetlands |
|  | 002000200200000 | 1042610 | 2.51 | Wetlands, cloud shadow, thin cloud |
|  | 002000200220000 | 2513475 | 6.05 | Forest, thin cloud |
|  | 002000220220000 | 5215137 | 12.56 | Forest, thin cloud, developed, grassland |
|  | 002200220220000 | 6863976 | 16.53 | Forest, barren land, grassland, thin cloud |
|  | 002200220222000 | 13570460 | 32.69 | Forest, shrub land, grassland |
|  | 002200222222000 | 4723985 | 11.38 | Forest, barren land, grassland, developed land |
|  | 002202220220000 | 60682 | 0.15 | Thin cloud |
|  | 002202222222000 | 37069 | 0.09 | Barren land |
|  | 002220222222000 | 3517290 | 8.47 | Forest |
|  | 002220222222200 | 530880 | 1.28 | Barren land, developed land |
|  | 002220222222220 | 54708 | 0.13 | Developed land |
|  | 002222222222000 | 257839 | 0.62 | Barren land |
|  | 002222222222200 | 486056 | 1.17 | Barren land |
|  | 022222222222000 | 42855 | 0.10 | Thin cloud, barren land |
|  | 022222222222200 | 221814 | 0.53 | Barren land, developed land |
|  | 222222222222200 | 41949 | 0.10 | Barren land, developed land |
| Total |  | 40976765 | 98.70 |  |
| LC81240512014030LGN00 | 000000000000000 | 241070 | 0.58 | Water |
|  | 000000200200000 | 63458 | 0.15 | wetlands |
|  | 002000200200000 | 173716 | 0.42 | wetlands |
|  | 002000200220000 | 278102 | 0.67 | Wetlands, Cloud shadow |
|  | 002000220220000 | 1068798 | 2.57 | Shadow, Forest |
|  | 002200220220000 | 2085190 | 5.01 | Forest, Shadow |
|  | 002200220222000 | 10979782 | 26.40 | Forest |
|  | 002200222222000 | 4414831 | 10.62 | Forest, Shrub land |
|  | 002202222222000 | 65173 | 0.16 | Developed land, Cloud |
|  | 002220222222000 | 8318551 | 20.00 | Shrub land, Grassland |
|  | 002220222222200 | 901276 | 2.17 | Barren land |
|  | 002220222222220 | 140485 | 0.34 | Barren land |
|  | 002222222222000 | 2365639 | 5.69 | Barren land, Shrub land , Thick cloud |
|  | 002222222222200 | 3092712 | 7.44 | Barren land, Grassland |
|  | 002222222222220 | 1234670 | 2.97 | Barren land, Developed land |
|  | 022222222222000 | 980222 | 2.36 | Barren land, Developed land, Grassland |
|  | 022222222222200 | 2691608 | 6.47 | Barren land, Developed land |
|  | 022222222222220 | 483761 | 1.16 | Barren land, Developed land |
|  | 200000000000000 | 160586 | 0.39 | Water |
|  | 202200220220000 | 35215 | 0.08 | Grassland, Developed land |
|  | 202200220222000 | 69882 | 0.17 | Grassland, Thin cloud |
|  | 202220222222000 | 179328 | 0.43 | Shrub land, Grassland, Forest |
|  | 222222222222000 | 197946 | 0.48 | Grassland, Shrub land, Barren land, cloud |
|  | 222222222222200 | 652262 | 1.57 | Barren land |
|  | 222222222222220 | 200366 | 0.48 | Barren land |
|  | 000000000000000 | 241070 | 0.58 | Water |
| Total |  | 41074629 | 98.77 |  |
|  | 000000000000000 | 4247018 | 10.21 | Water |
| LC81240522014046LGN00 | 000000200200000 | 60893 | 0.15 | Water, cloud shadow |
|  | 002000200200000 | 133033 | 0.32 | Wetlands, cloud shadow |
|  | 002000200220000 | 210399 | 0.51 | Wetlands, cloud shadow |
|  | 002000220220000 | 805420 | 1.94 | Forest, grassland, wetlands, thin cloud |
|  | 002200220220000 | 5770519 | 13.88 | Forest, grassland, Wetlands, thin cloud |
|  | 002200220222000 | 12104829 | 29.11 | Forest, shrub land |
|  | 002200222222000 | 4095455 | 9.85 | Shrub land, thin cloud, Developed land |
|  | 002220222222000 | 6128662 | 14.74 | Shrub land, Developed land, Thin cloud |
|  | 002220222222200 | 866822 | 2.08 | Barren land, Developed land, |
|  | 002222222222000 | 1182658 | 2.84 | Barren land, Developed land, Cloud |
|  | 002222222222200 | 1631302 | 3.92 | Barren land, Developed land |
|  | 002222222222220 | 119485 | 0.29 | Developed land, Barren land |
|  | 022222222222000 | 466472 | 1.12 | Barren land, Developed land, Cloud |
|  | 022222222222200 | 1456403 | 3.50 | Barren land |
|  | 022222222222220 | 145827 | 0.35 | Barren land |
|  | 202220222222000 | 91062 | 0.22 | Shrub land |
|  | 222222222222000 | 190583 | 0.46 | Developed land, Cloud, Barren land |
|  | 222222222222200 | 782381 | 1.88 | Barren land |
|  | 222222222222220 | 267367 | 0.64 | Barren land |
| Total |  | 40756590 | 98.01 |  |

Land cover types associated with each SRC modulation pattern in Table 2 are result of visual interpretation. For ilustration, we select some SRC modulation patterns for each Landsat 8 OLI scenes. Figure 4 show SRC modulation patterns image for a window arround Hanoi City extracted from the scene LC81270452013336LGN00.

(a) 			   (b)			       (c) 				(d)

Figure 4. (a) short wave infrared color composite RGB: 5, 4, 3; (b) pattern 000000000000000 - water;

(b) pattern 002200220220000 - vegetation; (d) pattern 002220222222200 – barren land, build up

This image window covers Hanoi City and surrounding areas. There are developed land, paddy field, rivers and lakes. Figure 4 (a) shows short wave infrared color composite by assigning band 5 to red, 4 to green and 3 to blue. In this color composite, water appears in bluish, vegetation in greenish tint and barren land in different grades of magenta. Spectral pattern 000000000000000 is presented in Figure 4 (b). We can observe that almost all water bodies including river, lakes, ponds and wetland are included in this pattern. Spectral pattern 002200220220000 is dominated by green vegetation and in this image window it is mainly agricultural cultivation. The pattern 002220222222200 involves barren land and built up, which is closely related to urban area of Hanoi City.

(a) 			   (b)			       (c) 				(d)

Figure 5. (a) short wave infrared color composite RGB: 5, 4, 3; (b) pattern 000000000000000 - water;

(c) pattern 002200220222000 – green vegetation; (d) pattern 022222222222200 – barren land, brown shrub

Figure 5 displays an extract from scene LC81240512014030LGN00 that covers an area of highland Tay Nguyen. Land cover of this area features evergreen vegetation on mountain, deciduous forest and shrub on plateaus land and paddy field around rivers and streams. Again, Figure 5 (a) is short wave infrared color composite RGB: 5, 4, 3. We can distinguish easily evergreen vegetation by greenish color, water by bluish tone and brown vegetation by magenta in different shades. Image of pattern 000000000000000 is presented in Figure 5 (b) shows large water bodies of the image window. Figure 5 (c) is image of pattern 002200220222000 gives distribution of green vegetation. Barren land and brown shrub (shrub during fall leaf season) is main content of Figure 5 (d), which is image of pattern 022222222222200.

(a) 			   (b)			       (c) 				(d)

Figure 6. (a) short wave infrared color composite RGB: 5, 4, 3; (b) pattern 000000000000000 - water;

(c) pattern 002200220222000 - forest; (d) pattern 022222222222200-bush and grassland

Figure 6 shows a portion of the scene LC81240522014046LGN00. Evergreen forest dominates landscape of this area. Images of patterns 000000000000000, 002200220222000 and 022222222222200 are presented in Figure 6 (a), 6 (b) and 6 (c) respectively. Evergreen forest and water body are closely related with patterns 002200220222000 and 000000000000000. Unlike Figure 5 (d), the pattern 022222222222200 in Figure 6 (d) shows bush and grassland.

4. DISCUSSION AND CONCLUSION

In this research, we grouped image pixels according to their SRC modulation patterns and tried to explain meaning of each modulation pattern by visual interpretation. There are hundreds of SRC modulation patterns in each Landsat 8 OLI scene. Frequency of patterns varies from some millions to one pixel. However, number of patterns with large enough frequency is quite small and about some tens only. There are 40,976,765 pixels in scene LC81270452013336LGN00 distributed in 559 SRC modulation patterns but only 19 patterns with highest frequency comprise 98.7% of total pixels. The scene LC81240512014030LGN00 has 41,591,459 pixels spreading over 725 SRC modulation patterns. In this scene, 98.6% of total pixels equivalent to 40,996,002 pixels are allocated in only 26 SRC modulation patterns. Similarly, number of significant SRC modulation patterns in the scene LC81240522014046LGN00 is 20 of 585. These 20 SRC modulation patterns contain 40756590 pixels equivalent to 98.0% of total pixels in this scene.

We found out by visual interpretation that each SRC modulation pattern is closely related to very limited number of land cover types.  Some patterns like 00000000000000, 002200220222000, 222222222222220 are interpreted consistently as water, green vegetation and barren land. The other patterns are mixture between water and vegetation or vegetation and barren land. Meaning of those patterns are stable in all three Landsat OLI scenes.

In conclusion, the paper demonstrated a new approach to understanding spectral signatures of Landsat 8 OLI image.  By grouping of pixel vectors to limited SRC modulation patterns, we can establish relation between each pixel to limited number of land cover types, which helps to easier and better land cover interpretation. We have found out interesting observation that meaning of SRC modulation patterns is stable in different Landsat 8 OLI scenes over different geographic regions. This finding could lay the foundation for development of new method for land cover classification.

ACKNOWLEDGEMENT

The authors wish to thank the Institute of Geography, Vietnam Academy of Science and Technology for funding this research.

REFERENCE

Canty, Morton J., Allan A. Nielsen, and Michael Schmidt. "Automatic radiometric normalization of multitemporal satellite imagery." Remote Sensing of Environment 91.3 (2004): 441-451.

Duong, N. D. (1997, October). Graphical analysis of spectral reflectance curve. In Proceedings of the 18th Asian Conference on Remote Sensing (pp. 20-24).

Laborte, Alice G., Aileen A. Maunahan, and Robert J. Hijmans. "Spectral signature generalization and expansion can improve the accuracy of satellite image classification." PloS one 5.5 (2010): e10516.

Olthof, Ian, Chris Butson, and Robert Fraser. "Signature extension through space for northern landcover classification: A comparison of radiometric correction methods." Remote Sensing of Environment 95.3 (2005): 290-302.

Ruby, Jeffrey G., and Robert L. Fischer. "Spectral signatures database for remote sensing applications." International Symposium on Optical Science and Technology. International Society for Optics and Photonics, 2002.

Shaw, Gary A., and Hsiao-hua K. Burke. "Spectral imaging for remote sensing."Lincoln Laboratory Journal 14.1 (2003): 3-28.

Thenkabail, P., et al. "Spectral matching techniques to determine historical land-use/land-cover (LULC) and irrigated areas using time-series 0.1-degree AVHRR Pathfinder datasets." Photogrammetric Engineering & Remote Sensing73.10 (2007): 1029-1040.